Complete Plate Fillet Welds Assessment

Name: ___________________________    				Date: _________

Type of Joint:  ____________________   Position of Fillet: ____________________

Welding process: __________________         Material and Size of base metal ______________

AWS classification of Filler Metal _____________    Size of filler metal: _________________

Voltage: _________   Wire feed Speed ___________ Amps __________  Shielding gas______

|  | LAP | T | CORNER | EDGE |
| --- | --- | --- | --- | --- |
| Surface slag has been removed from 99% of the joints and surrounding area? | yes/no | yes/no | yes/no | yes/no |
| Surface spatter or whisker has been removed from 99% of the joints and surrounding area? | yes/no | yes/no | yes/no | yes/no |
| Free of stray arc strikes | yes/no | yes/no | yes/no | yes/no |
| *0 Arc Strikes |  |  |  |  |
| Are joints free from linear misalignment? | yes/no | yes/no | yes/no | yes/no |
| *Allow 1/8" maximum from the metal edge |  |  |  |  |
| Tie-ins at corners are smooth and continuous to within | yes/no | yes/no | yes/no | yes/no |
| *1/8" maximum |  |  |  |  |
| All stop/restarts smooth on the capping layer | yes/no | yes/no | yes/no | yes/no |
| *1/16" variation |  |  |  |  |
| Fillet joints completely fused into the parent materials? | yes/no | yes/no | yes/no | yes/no |
| *no overlap/cold lap |  |  |  |  |
| Fillet joints completely free from surface porosity | yes/no | yes/no | yes/no | yes/no |
| *Any porosity over ⅛” is a fail, *Allowed up to ⅜” porosity total per inch is a fail |  |  |  |  |
| Fillet joints free from undercut? | yes/no | yes/no | yes/no | yes/no |
| *1/32" maximum |  |  |  |  |
| Fillet joint weld sizes in accordance with the specifications and drawings? | yes/no | yes/no | yes/no | yes/no |
| Correct bead height and width |  |  |  |  |
| * + or - 1/16" variation", 2 times the electrode width, specified width and height |  |  |  |  |
| TOTAL OF YES MARKS |  |  |  |  |
| Lap welds must cover the edge of the base metal |  |  |  |  |