Welding 1: Surface Weld Assessment

Name: ___________________________    				Date: _________

Type of weld:  ____________________   Position of Fillet: ____________________

Welding process: __________________         Material and Size of base metal ______________

AWS classification of Filler Metal _____________    Size of filler metal: _________________

Voltage: _________   Wire feed Speed ___________ Amps ________  Shielding gas________

| Surface slag has been removed from 99% of the joints and surrounding area? | yes/no |
| --- | --- |
| Surface spatter or wisker has been removed from 99% of the joints and surrounding area? | yes/no |
| Free of stray arc strikes | yes/no |
| Joints are free from linear misalignment? | yes/no |
| *Allow 1/8" maximum from the metal edge |  |
| Tie-ins at toes are smooth and continuous to within | yes/no |
| *1/8" maximum |  |
| All stop/restarts smooth on the capping layer | yes/no |
| *1/16" variation |  |
| Fillet joints completely fused into the parent materials? | yes/no |
| *no overlap/cold lap/cold start |  |
| Fillet joints completely free from surface porosity | yes/no |
| *1/4" maximum of porosity |  |
| Fillet joints free from undercut? | yes/no |
| *1/16" maximum |  |
| Fillet joint weld sizes in accordance with the specifications and drawings? | yes/no |
| Correct bead height and width |  |
| * + or - 1/16" variation", 2 times the electrode width, specified GMAW width and height |  |
| TOTAL OF YES MARKS |  |